# IoT Based EB Line Monitoring, Fault Detection and Electricity Theft Detection System

## Abstract

This project presents a 12 V prototype of an Electricity Board (EB) line monitoring system using an ESP32. The system continuously monitors voltages at Pole A, Pole B, and Pole C using DC voltage sensor modules, measures line current using an ACS712 current sensor, detects line-cut faults, low-voltage conditions, and possible electricity theft, and displays live data on a local web dashboard using WebSocket communication. A relay disconnects the load only during line-cut faults.

## Objectives

Monitor voltage at Pole A, Pole B and Pole C.

Detect line cut between Pole A-B and Pole B-C.

Detect low voltage conditions.

Detect possible electricity theft using current sensing.

Display live data on a local HTML dashboard.

Automatically disconnect the load during line-cut faults.

## Hardware Components

| Component | Qty | Purpose |
| --- | --- | --- |
| ESP32 Dev Board | 1 | Main controller and Wi-Fi |
| 0-25V Voltage Sensor Module | 3 | Pole voltage monitoring |
| ACS712 Current Sensor | 1 | Current measurement |
| 5V Relay Module | 1 | Disconnect load during faults |
| PN2222A Transistor | 1 | Relay driver |
| Flyback Diode (1N4007) | 1 | Relay protection |
| 12V DC Supply | 1 | Prototype supply |
| LED | 3 | Pole status indication |
| 4.7kΩ Resistor | 3 | LED current limiting |
| Breadboard/Jumper Wires | 1 set | Connections |

## Software Used

Arduino IDE

ESP32 Board Package

HTML, CSS, JavaScript

WebSocket Library

WiFi Library

## Working Principle

The 12 V supply feeds the prototype line through Pole A, Pole B and Pole C. Three voltage sensors measure the voltage available at each pole. An ACS712 current sensor measures the line current. The ESP32 compares the measured values with preset thresholds.

If Pole A has voltage but Pole B does not, the system reports 'Line Cut Between Pole A and Pole B' and turns OFF the relay.

If Pole B has voltage but Pole C does not, the system reports 'Line Cut Between Pole B and Pole C' and turns OFF the relay.

If voltage is below the threshold, the dashboard shows 'Low Voltage'.

If voltage is present but current is nearly zero, the system reports 'Electricity Theft / No Load' while keeping the relay ON.

All measurements are sent in real time to a local HTML dashboard using WebSockets.

## Block Diagram

12V DC Supply
      |
  ACS712 Current Sensor
      |
   Pole A -- Voltage Sensor --> ESP32
      |
   Pole B -- Voltage Sensor --> ESP32
      |
   Pole C -- Voltage Sensor --> ESP32
      |
    Relay --> Load

ESP32 --> Wi-Fi --> Local Web Dashboard (HTML + WebSocket)

## Fault Logic

| Condition | Message | Relay |
| --- | --- | --- |
| A OK, B=0V | Line Cut A-B | OFF |
| B OK, C=0V | Line Cut B-C | OFF |
| Voltage below threshold | Low Voltage | ON |
| Voltage present, current≈0 | Electricity Theft | ON |
| All normal | System Normal | ON |

## Advantages

Real-time monitoring

Fast fault location

Low-cost prototype

Local dashboard without cloud

Automatic relay protection

Easy to expand to more poles

## Applications

Electricity Board demonstration

Smart grid research

Distribution line monitoring

Educational embedded systems projects

Industrial DC power monitoring

## Future Scope

Upgrade to real 230 V AC using isolated AC sensors.

Add GPS location for fault reporting.

Store historical data in a database.

SMS/Telegram notifications.

Support more poles and multiple feeders.

## Conclusion

The proposed IoT-based EB line monitoring system demonstrates reliable detection of line-cut faults, low-voltage conditions, and possible electricity theft in a safe 12 V prototype. The ESP32 and local web dashboard provide real-time monitoring and automatic protection, making the project suitable for embedded systems demonstrations and future smart-grid applications.